# Large-Scale Direct SLAM

- This guide demonstrates how to use the monocular LSD SLAM ROS package in combination with a wide field-of-view camera. Make sure to follow the installation instructions first! For more information on the algorithm, check https://github.com/tum-vision/lsd_slam. For a brief demonstration, watch the lsd_slam_tutorial.mp4 video in this portfolio.

## Usage

- Connect a webcam to your laptop. LSD SLAM works best with a wide field-of-view camera, so we use the Genius WideCam F100 which has a field-of-view of 120°.

- In the folder '/lsd_slam/lsd_slam_core/calib', create a new file named 'widecam_calib.cfg'. Copy and paste the following lines:
	0.527334 0.827306 0.473568 0.499436 0
	640 480
	none
	640 480
These are the distortion parameters for the camera.

- Execute 'roslaunch gscam gscam_640_480.launch'. This launches an instance of the open source gscam webcam driver. The images are subsampled to a resolution of 640x480, which works best for Monocular LSD SLAM.

- Open the LSD SLAM point cloud viewer by executing 'rosrun lsd_slam_viewer viewer'.

- Run LSD SLAM by executing 'rosrun lsd_slam_core live_slam /image:=/camera/image /camera_info:=/camera/camera_info _calib:=widecam_calib.cfg'. This starts an instance of lsd_slam, that listens to the /camera/image and /camera/camera_info ROS topics. It will also use the distortion parameters that we have set in widecam_calib.cfg. A window will pop up, showing the camera image.

- The LSD-SLAM parameters can be set dynamically by running 'rosrun rqt_reconfigure rqt_reconfigure', and selecting LSD SLAM. The most important parameters are KFUsageWeight and KFDistWeight. These parameters should be changed empirically, depending on the environment that has te be mapped. High values will result in an accurate trajectory, but generate a high amount of noise. It is best to use low values for these parameters, so that the amount of noise will stay limited.
Also, cameraPixelNoise and minUseGradient should be changed empirically until an adequate result is obtained. These parameters indicate the camera image noise.

- Going around the room in a circle with the camera always pointed at the middle of the room will render the best result.